Example Agenda for Pre-Galvanizing Meeting Related to AESS Projects

- AESS Category Characteristics & Cost Matrix

  - Review modified AESS matrix for HDG and AGA publication Hot-Dip Galvanized Architecturally Exposed Structural Steel:  Specifiers Guide

  - Supply of project drawings containing information on weight and surface area

  - What modified AESS category applies to which pieces/assemblies

  - Define custom characteristics for HDG for each AESS category

  - Identify critical or significant surfaces

  - Review AISC/CISC cost matrix for AESS

  - Rank category characteristics in order of importance regarding cost

  - Identify any critical parts or surfaces where budget should be focused

- Identification & Marking of Parts

  - Identify acceptable types of markings

  - How to mark AESS category and characteristics required for each part

  - How to mark critical or significant surfaces

- Steel Chemistry

  - Identify all steel grades used and for what parts

  - Supply of Material Test Reports (MTRs) for all heats of steel

  - Identify assemblies where mixed appearance may occur

  - Identify steels where excessive coating thickness and/or brittle coatings may occur

- Design & Fabrication

  - Evaluate susceptibility for warpage/distortion (ASTM A384)

  - Evaluate susceptibility for embrittlement (ASTM A143)

  - Confirm the use of design and fabrication best practices (ASTM A385 and AGA Publication Design of Products to be HDG After Fabrication)

  - Identify vent/drain holes where size or location can be optimized for aesthetics

  - Acceptable appearance of vent/drain holes before HDG (drilled and/or smoothed)

  - Evaluate susceptibility for weeping welds or rust bleeding; identify mitigation strategy

  - Identify all welding rod types used and for what assemblies

  - Supply of Material Test Reports (MTRs) for all welding wire/rods used

  - Use of anti-spatter sprays and acceptable materials

  - Acceptable appearance of welds before galvanizing

  - Use of masking materials

  - Evaluate oversized parts and acceptability of progressive dipping

  - Define a procedure for communicating and resolving inadequate design details or insufficient preparation of welds, vent/drain holes, etc. after delivery to the galvanizer

- HDG Processing

  - Define a procedure to communicate and remedy cosmetic damages due to transportation or handling before HDG

  - Evaluate available lift points for HDG and any potential impact on aesthetics

  - Use of post-treatments

- Finishing & Surface Smoothing

  - Acceptable appearance after smoothing for handling marks, roughness, runs, spikes, dross inclusions, galvanizing skimmings, or other surface conditions

  - Acceptable appearance of HSS seams

  - Acceptable appearance of mill markings

  - Plugging of holes, plug material, and acceptable appearance of plugged holes

  - Acceptable appearance of welds; agree on a procedure for smoothing raised welds

  - Acceptable appearance of welds, seams, mill markings, or other raised areas where grinding prior to HDG was not performed; agree on a procedure should this issue arise

  - Acceptable appearance of thermally cut edges on thick steels; agree on a procedure should this issue arise

  - Acceptable appearance of progressive dip line

- Duplex Systems

  - Define acceptance criteria and assign responsibilities for surface smoothing. See ASTM A123 6.2, A123 6.4, and D6386 5.2 (for painting) or D7803 5.1.1 (for powder coating)

  - If using zinc-rich paint for touch-up and repair at the galvanizing plant, evaluate re-coat window of the paint and mutually agree whether repair material should be applied by the galvanizer or painter

  - Define use of compatible filler materials to fill small indentations in the galvanizing

- Handling, Storage & Wet Storage Stain (WSS)

  - Define acceptable handling methods after HDG

  - Define packaging or dunnage requirements

  - Define realistic expectations for preventing and removing WSS at the HDG plant

  - Responsibility for preventing WSS during trucking

  - Define storage and WSS removal procedures at the job site

- Inspection

  - Define customer hold-points during QA/QC

  - Reporting

- Touch-Up & Repair At The Galvanizing Plant

  - Repair material selection and practical limitations of the chosen material

  - Use of cosmetic touch-up

  - Cosmetic touch-up material selection and color matching

- Touch-Up & Repair At The Jobsite

  - Repair material selection and practical limitations of the chosen material

  - Define responsibility and a procedure for field repairs due to transportation, handling damage (flaking), or other erection activities

Define responsibility and a procedure for remedying weeping welds and rust bleeding

galvanizeit.org